P

Pear Skin

Industry jargon.

The "fuzzy" appearance on the load-bearing surface of bearing races. Caused by contamination trapped between the races and the rolling elements.

Pearlite

A ferrite and cementite metastable lamellar aggregate which results from the transformation of austenite at a temperature above the bainite range.

Pfund Hardness Number

A reference standard for the hardness of a coating on a material.

A measurement of the indentation created in a coating when a standardized round-tip quartz or sapphire stylus is applied under a specific pressure.

Pickling

The removal of surface oxides by chemical bath or electrochemical reaction.

Pierce, Winslow S., Jr.

Developed and later patented, in 1924, a superior system for producing precision split ball bearing inner races without loss of material. 

Mr. Pierce later went on to develop the miniature ball bearings used in the Norden bombsight, critical to the allied air war effort during World War II.

Plastic Deformation

See also Elastic Deformation.

The permanent deformation of a material when stress is applied. For plastic deformation to occur, the material must be stressed beyond its elastic limit.

Powder Metallurgy

The use of metal powders under heat and pressure to produce shaped objects in a cavity as an alternative to machining or other metalforming options. Powder metal parts are most often net or near-net shapes.

A common application of powder metal parts in the bearing industry is for clutch release bearing sleeves.